How to convert original record files into AVI format?

- Click the player icon by right key of mouse. And run it by administrator.

- Import recorded video file in player list.

.

- Click set up icon and then “convert AVI”

- Please set up the convert file time less than 1 hour. See below picture, if the start time is 4:46:31, please set the stop time earlier than 5:46:30. So, the file length will less than 1 hour. And then, click start.

Attention: Please do not change the output folder or it may cause unexpected problem.

- After convert finished, go to the folder to got the converted files.

Attention: if the converted files can not be played back, try to convert it again.

KEY POINT:

- Run player by administrator

- Set convert file length less than 1 hour. In default setting, if there has 6 imported files and the total recording time is 8 hours, the converted file length will be 8 hours too, but it has only 1 file. In this case, the converted file may have problem such as convert failed, or recording time, frame lost.